RAVBSW gPTP

18 Dec 2014

Introduction

IEEE802.1AS gPTP is implemented on the Renesas AVB Switch (RAVBSW) using linuxptp, which forms part of Open-AVB. linuxptp is used on Renesas AVB End Stations.

Changes Made To linuxptp version 1.4

The source code has been changed to add Transparent Clock functionality and a new config file option, tcMode. This option takes effect only when linuxptp is started with multiple interfaces.

Changes:

- Forwarding of ANNOUNCE messages, retaining the original values for fields in the messages.

- Forwarding pairs of SYNC & FOLOW_UP messages, including the calculation, addition and insertion of the correction field into the FOLOW_UP.

- Participation in the exchange of PATH_DELAY messages with all connected nodes.

- Additions to MANAGEMENT messages & interface.

  - CLOCK_DESCRIPTION now includes the clock type.

  - TIME_PROPERTIES_DATA_SET now includes the clock mode

  - PORT_DATA_SET_NP output has been modified to include the port name

Note that boundary clock functionality remains unchanged if tcMode is not set.

Other Changes

linuxptp requires a gPTP Linux device. On the ZC706 platform that RAVBSW uses, that is provided by the ravbsw-ptp kernel module.

In order to retain proper functionality of the ptp clock, the increment value (loaded into the GTI register at start up) has been modified in the kernel module ravbsw, to be suitable for the ZC706’s 100MHz clock.

Restrictions

- When tcMode is set, linuxptp is fixed into Transparent Clock mode and cannot change to Grand Master.

- The new makefile is specific to a buildroot target environment with kernel 3.15 headers.

Build

A new makefile, makefile.ravbsw, builds the executables specifically for the RAVBSW buildroot target environment all the software runs in. This currently uses hard-coded paths for the target toolchain and kernel headers.

$ make –f makefile.ravbsw clean

$ make –f makefile.ravbsw

Installation

$ make -f makefile.ravbsw install

This will copy the executables (ptp4l, pmc, phc2sys & hwstamp_ctl) to the target install path /tftpboot/ZC706/usr/bin/ (this is specific to the ZC706, using a buildroot filesystem and a host NFS filesystem)

Configuration File

gmCapable and tcMode are mutually exclusive. Priority1 and priority2 are user-settable, allowing the user to choose who will be master. Generally a slave only node will have these two fields as 255. A gmCapable node will have priority1 as 128 (higher priority).

Example configuration file options:

[global]

#

# Default Data Set

#

twoStepFlag			1

gmCapable			0

tcMode			1

priority1			128

priority2			128

domainNumber		0

clockClass			248

clockAccuracy		0xFE

offsetScaledLogVariance	0xFFFF

free_running		0

freq_est_interval		1

#

# Port Data Set

#

logAnnounceInterval	1

logSyncInterval		-3

logMinPdelayReqInterval	0

announceReceiptTimeout	3

syncReceiptTimeout	3

delayAsymmetry		0

fault_reset_interval	4

neighborPropDelayThresh	200000

#

# Run time options

#

assume_two_step		1

logging_level		6

path_trace_enabled	1

follow_up_info		1

tx_timestamp_timeout	1

use_syslog			1

verbose			0

summary_interval		0

kernel_leap			1

check_fup_sync		0

Usage

The RAVBSW kernel modules ravbsw & ravbsw_ptp must be loaded if these are not statically linked into the kernel.  To enable Transparent Clock mode, tcmode must be set in the config file and ptp4l must be started with at least two Ethernet interfaces. Eg,

ptp4l

This is the gPTP service and normally runs in the background.

$ ptp4l -i avb0 -i avb1 -p /dev/ptp0 -f /config_file.cfg

Sample config files have been provided

- ravbsw_master.cfg (Sets priority1 and priority2 to 128. gmCapable. A priority of 128 represents a master-capable device)

- ravbsw_slave.cfg (Priority1 255, priority2 255)

- ravbsw_mid.cfg (Priority1 128, priority2 255. gmCapable. This is the ideal configuration for a Transparent Clock)

PMC

This is a management client allowing access to various variables during ptp execution. The following are useful commands to run while ptp4l is running:

$ pmc -u -b 1 'GET TIME_STATUS_NP'

$ pmc -u -b 1 'GET PORT_DATA_SET'

$ pmc -u -b 1 'GET PORT_DATA_SET_NP'

$ pmc -u -b 1 'GET CLOCK_DESCRIPTION'

Example outputs:

$ pmc -u -b 1 'GET PORT_DATA_SET_NP’

sending: GET PORT_DATA_SET_NP

020000.fffe.000082-1 seq 4 RESPONSE MANAGMENT PORT_DATA_SET_NP

neighborPropDelayThresh 10000

asCapable               1

PortName                avb0

020000.fffe.000082-3 seq 4 RESPONSE MANAGMENT PORT_DATA_SET_NP

neighborPropDelayThresh 10000

asCapable               1

PortName                avb3

$ pmc -u -b 1 'GET TIME_STATUS_NP'

sending: GET TIME_STATUS_NP

020000.fffe.888802-0 seq 0 RESPONSE MANAGMENT TIME_STATUS_NP

master_offset              0

ingress_time               0

cumulativeScaledRateOffset +1.000000000

scaledLastGmPhaseChange    0

gmTimeBaseIndicator        0

lastGmPhaseChange          0x0000'0000000000000000.0000

gmPresent                  false

gmIdentity                 020000.fffe.888802

$ pmc -u -b 1 'GET CLOCK_DESCRIPTION'

sending: GET CLOCK_DESCRIPTION

020000.fffe.888802-1 seq 0 RESPONSE MANAGMENT CLOCK_DESCRIPTION

clockType             0x8000 (TC)

physicalLayerProtocol IEEE 802.3

physicalAddress       02:00:00:88:88:02

protocolAddress       3 02:00:00:88:88:02

manufacturerId        00:00:00

productDescription    ;;

revisionData          ;;

userDescription

profileId             00:1b:19:00:02:00

$ pmc -u -b 1 'GET PORT_DATA_SET'

sending: GET PORT_DATA_SET

020000.fffe.888802-1 seq 0 RESPONSE MANAGMENT PORT_DATA_SET

portIdentity            020000.fffe.888802-1

portState               MASTER

logMinDelayReqInterval  0

peerMeanPathDelay       37992

logAnnounceInterval     1

announceReceiptTimeout  3

logSyncInterval         -3

delayMechanism          2

logMinPdelayReqInterval 0

versionNumber           2

A full manual can be found at:

linux.die.net/man/8/pmc

Or by running

$ pmc –h